# ICM – Computer Science Major

# M1 Cyber Physical and Social Systems

# Course unit on Data Interoperability and Semantics

# Tutorial 1

# 1) Bluetooth Mesh Model

The definitions below are extracts of a Bluetooth standard. Complete the tables.

Bluetooth Mesh Model specification v1.0.1 Section 3.1.6.1 Generic Battery Level (modified)

The Generic Battery Level state is a value ranging from 0 percent through 100 percent. The values must be divided by two (ex., 0x03 stands for 1.5 %). Values above 0xC8 (100 %) are forbidden.

|  | encoded | decoded |
| --- | --- | --- |
| 8-bit unsigned integer (UINT8) | 0xA7 (hexa) 0b 1010 0111 = 167 | 83.5 % |
| UINT8 | 0b01010110 (binary) 64+16+4+2 = 86 | 43 % |

Bluetooth Mesh Model specification v1.0.1 Section 3.1.7.6 Local Altitude

The Local Altitude field determines the altitude of the device relative to the Generic Location Global Altitude. This is a 16-bit signed integer in decimeters. The valid range is from -32768 decimeters (0x8000) through 0 decimeters (0x0000) up to 32765 decimeters (0x7FFD).

The following formula can be used to decode data:  B2TX=-xw-1∙2w-1+i=0w-2xi∙2i

|  | encoded | decoded |
| --- | --- | --- |
| INT16 - Big Endian (AB) | 0x8001 | We know 0x8000 = -32768 dm +1 -32767 dm |
| INT16 - Little Endian (BA) | 0x8001 0x0180 0b 0000 0001 1000 0000 | 256 + 128 = 384 dm |

# 2) IEEE 754

Decode the following hexadecimal strings using the Single precision IEEE 754 floating-point standard:

you can try with https://www.h-schmidt.net/FloatConverter/IEEE754.html

- 0x3F000000

3F 00 00 00 16  = 0011 1111 00…2  // first convert to binary

= 0 0111 1110 0…2   // group bits differently

S = 0 	E = 1000 00002 - 210 = 128 – 2 = 12610 	M = 0 	// extract sign, exponent, mantissa

apply formula: -10 x 2(126-127) x 1.0 = 0.5 10

b) 0x44800000

44 80 00 00 16 	= 0100 0100 1000 00.. 2 // first convert to binary

= 0 1000 1001 0.. 2 // group bits differently

S = 0 	E = 1000 1001 2 = 128 + 8 + 1 = 13710	M = 0  // extract sign, exponent, mantissa

apply formula: -10 x 2(137-127) x 1.0 = 210 = 1024 10

Encode the following numbers using the Single precision IEEE 754 floating-point standard:

c) -3.0

-3.0 	= -1 x 2 x 1.5  		// in the form of "sign" x "power of two" x "number between 1 and 2"

= (-1)1 x 2(128 – 127) x (1 + 2-1) = (-1)1 x 2(128 – 127) x 1.12  	// IEEE 574 floating point formula

S = 1	E = 12810 = 1000 0000 2 	M = 1000…2 		// explicit components

0 1000 0000 100…2 = 0100 0000 0100 00…2  = 40 40 00 00 16

d) 24.0

24.0	= 1 x 16 x 1.5 		// in the form of "sign" x "power of two" x "number between 1 and 2"

= (-1)0 x 2(131 - 127) x 1.12	// IEEE 574 floating point formula

S = 0 	E = 13110 = 128 + 2 + 1 = 1000 00112 	M = 1000..2 	// explicit components

0 1000 0011 1000..2 = 0100 0001 1100 0…2 = 41 C0 00 00 16

# 3) UTF-8

Convert the following strings to UTF-8:

- cs

This is two characters ! Not a strange, unique character

Code point for 'c' is U+0063, code point for 's' is U+0073.

Both are encoded on one byte only.

So the encoding of "cs" is 63 7316

check with command echo -n cs | od -t x1

- ¥

Code point is U+00A5 (1010 01012)

Encoded on two bytes, with mask 110xxxxx 10xxxxxx

So the encoding is 11000010 101001012 = C2 A5 16

check with command echo -n ¥ | od -t x1

- €

Code point is U+20AC (0010 0000 1010 11002)

Encoded on three bytes as per the UTF-8 spec, with mask  1110xxxx 10xxxxxx 10xxxxxx

so the encoding is 11100010 10000010 101011002 = E2 82 AC 16

check with command echo -n € | od -t x1

- 😃

Code point is U+1F603 (0001 1111 0110 0000 00112)

Encoded on four bytes as per the UTF-8 spec, with mask  11110xxx 10xxxxxx 10xxxxxx 10xxxxxx

so the encoding is 11110000 10011111 10011000 100000112 = F0 9F 98 83 16

check with command echo -n 😃 | od -t x1

# 4) CBOR

Below is a document encoded using the Concise Binary Object Representation.

Write a JSON equivalent of this document.

Spaces and new lines in the representation are just there to help you.

82 A3 61 76 FA 3F800000 62 6363 63 455552 62 6373 63 E282AC

A3 61 76 FA 40F80000 62 6363 63 434E59 62 6373 62 C2A5

I will use notation x_y with x the major CBOR type encoded on 3 bits, and y the reminder on 5 bits.

let's decode byte by byte:

8216 = 4_2 , announces an array of two items.

A316 = 5_3 , announces an object of three key-value pairs

6116 = 3_1 , announces an UTF-8 text string encoded on 1 byte

7616 is the UTF-8 text string. A quick look up to table 1 gives character "v" – this is the first key

FA16 = 111_11010 = 7_26, announces a IEEE 754 Single-Precision Float  (next 32 bits = 4 bytes)

3F80000016 is an IEEE 754 Single-Precision Float

= 0011 1111 10…2  // first convert to binary

= 0 0111 1111 0…2   // group bits differently

S = 0 	E = 12710 	M = 0 	// extract sign, exponent, mantissa

apply formula: -10 x 2(127-127) x 1.02 = 1.0 10 – this is the first value

6216 = 3_2, announces an UTF-8 text string encoded on 2 bytes

636316 is the UTF-8 text string. A quick look up to table 1 gives characters "cc"– this is the second key

6316 = 3_3, announces an UTF-8 text string encoded on 3 bytes

45555216 is the string: "EUR" – this is the second value

6216 = 3_2, announces an UTF-8 text string encoded on 2 bytes

637316 is the string:  "cs" – this is the third key

6316  = 3_3, announces an UTF-8 text string encoded on 3 bytes

E282AC16  is the string "€" (see exercise 3c above) – this is the third value

so first object in JSON is {"v": 1.0, "cc": "EUR", "cs": "€"}

let's do the same for the second object:

first key is "v"

first value is IEEE 754 Single-Precision Float 40F8000016

= 0100 0000 1111 1000 00…2 	// first convert to binary

= 0 1000 0001 1111 0…2   	// then group bits differently

S = 0 	E = 128 + 110 = 129 	M = 1111 0…2 	// extract sign, exponent, mantissa

apply formula: -10 x 2(129-127) x 1.11112

= 410 x 1.11112   ("4 times 1 dot something)

// looks lengthy to compute 1 + 0.5 + 0.25 + 0.125 + 0.0625. let's transform that a bit

= 2 x 11.1112 ("2 times 3 dot something)

= 111.112  //  the entire part is 1112 = 7 , the fraction part is 0.5 + 0.25

= 7.7510

second key is "cc"

second value is "CNY"

third key is "cs"

third value is "¥"

In the end, we get JSON document

[{"v": 1.0, "cc": "EUR", "cs": "€"}, {"v": 7.75, "cc": "CNY", "cs": "¥"}]

check on cbor.me:

82                # array(2)

A3             # map(3)

61          # text(1)

76       # "v"

FA 3F800000 # float

62          # text(2)

6363     # "cc"

63          # text(3)

455552   # "EUR"

62          # text(2)

6373     # "cs"

63          # text(3)

E282AC   # "€"

A3             # map(3)

61          # text(1)

76       # "v"

FA 40F80000 # float

62          # text(2)

6363     # "cc"

63          # text(3)

434E59   # "CNY"

62          # text(2)

6373     # "cs"

62          # text(2)

C2A5     # "¥"

# 5) Base32 decoding

Below is the RFC 4648 Base32 Alphabet

Decode the following string: 	JVSWK5BAJJXWKICAGRYG2II=

you can test with echo JVSWK5BAJJXWKICAGRYG2II= | base32 -d

simple look up in the alphabet

9,    21,   18,   22,   10,   29,   1,    0,    9,    9,    23,   22,   10,

01001 10101 10010 10110 01010 11101 00001 00000 01001 01001 10111 10110 01010

8,    2,    0,    6,    17,   24,   6,    26,   8,    8

01000 00010 00000 00110 10001 11000 00110 11010 01000 01000

Groups of 8 bits:

01001101 01100101 01100101 01110100 00100000 01001010 01101111 01100101

4   D    6   5    6   5    7   4    2   0    4   6    6   7    6   5

0x4D=M   0x65=e   0x65=e   0x74=t   0x20=spce0x46=J   0x67=o   0x65=e

00100000 01000000 00110100 01110000 01101101 00100001 000 (pad = : we forget about the three last zeros)

2   0    4   0    3   4    7   0    6   D    2   1

0x20=spce0x40=@   0x34=4   0x70=p   0x6D=m   0x21=!

The answer is:

Meet Joe @4pm!

# 6) CRLF

Different OSs represent ends of line differently.

Microsoft Windows uses CRLF, Unix uses only LF, Classic MacOS used CR, then LF since Mac OS X.

a) What is the full name of CR and LF? Carriage Return and Line Feed

b) What are their ASCII codes? simple lookup in Annex 3: CR: 0D16  LF: 0A16

c) With what escape characters can these characters be included in a JSON string?  \r and \n

d) Why is it problematic that different OSs use different end-of-line characters?

It causes inconsistencies when sharing or processing text files across systems. For example, files may appear as a single long line or have extra blank lines if applications do not correctly handle the end-of-line convention. This is an interoperability problem! (in the title of the course )

# Appendix A: Single precision IEEE 754 floating-point standard

As an example, the value for number 0xbfc00000 is -1.5

# Appendix B: UTF-8 Encoding

UTF-8 encodes code points in one to four bytes, depending on the value of the code point. In the following table, the x characters are replaced by the bits of the code point:

| Code point ↔ UTF-8 conversion |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| First code point | Last code point | Byte 1 | Byte 2 | Byte 3 | Byte 4 |
| U+0000 | U+007F | 0xxxxxxx |  |  |  |
| U+0080 | U+07FF | 110xxxxx | 10xxxxxx |  |  |
| U+0800 | U+FFFF | 1110xxxx | 10xxxxxx | 10xxxxxx |  |
| U+10000 | U+10FFFF | 11110xxx | 10xxxxxx | 10xxxxxx | 10xxxxxx |

The first 128 code points (ASCII) need one byte. The next 1,920 code points need two bytes to encode, which covers the remainder of almost all Latin-script alphabets, and also IPA extensions, Greek, Cyrillic, Coptic, Armenian, Hebrew, Arabic, Syriac, Thaana and N'Ko alphabets, as well as Combining Diacritical Marks. Three bytes are needed for the remaining 61,440 code points of the Basic Multilingual Plane (BMP), including most Chinese, Japanese and Korean characters. Four bytes are needed for the 1,048,576 code points in the other planes of Unicode, which include emoji (pictographic symbols), less common CJK characters, various historic scripts, and mathematical symbols.

A "character" can take more than 4 bytes because it is made of more than one code point. For instance a national flag character takes 8 bytes since it is "constructed from a pair of Unicode scalar values" both from outside the BMP.

Encoding process

In these examples, character ‘|’ indicate how bits from the code point are separated and distributed among the UTF-8 bytes. Additional bits added by the UTF-8 encoding process are underlined.

- The Unicode code point for the euro sign € is U+20AC.

- As this code point lies between U+0800 and U+FFFF, this will take three bytes to encode.

- Hexadecimal 20AC is binary 0010 | 0000 10|10 1100. The two leading zeros are added because a three-byte encoding needs exactly sixteen bits from the code point.

- Because the encoding will be three bytes long, its leading byte starts with three 1s, then a 0 (1110...)

- The four most significant bits of the code point are stored in the remaining low order four bits of this byte (11100010), leaving 12 bits of the code point yet to be encoded (...0000 10|10 1100).

- All continuation bytes contain exactly six bits from the code point. So the next six bits of the code point are stored in the low order six bits of the next byte, and 10 is stored in the high order two bits to mark it as a continuation byte (so 10000010).

- Finally the last six bits of the code point are stored in the low order six bits of the final byte, and again 10 is stored in the high order two bits (10101100).

The three bytes 11100010 10000010 10101100 can be more concisely written in hexadecimal, as E2 82 AC.

# Appendix C: Some Unicode code points

Basic Latin Range: U+0000 – U+007F		Latin-1 Supplement: U+0080 – U+00FF

Currency Symbols U+20A0 – U+20C0		Emoji symbols U+1F600 – U+1F64F

# Appendix D: RFC 8949 - Concise Binary Object Representation (CBOR)

Concise Binary Object Representation (CBOR) is a binary data serialization format loosely based on JSON authored by C. Bormann. Like JSON it allows the transmission of data objects that contain name–value pairs, but in a more concise manner. This increases processing and transfer speeds at the cost of human readability. It is defined in IETF RFC 8949

Specification of the CBOR encoding

CBOR encoded data is seen as a stream of data items. Each data item consists of a header byte containing a 3-bit type and 5-bit short count. This is followed by an optional extended count (if the short count is in the range 24–27), and an optional payload.

For types 0, 1, and 7, there is no payload; the count is the value. For types 2 (byte string) and 3 (text string), the count is the length of the payload. For types 4 (array) and 5 (map), the count is the number of items (pairs) in the payload.

CBOR cheatsheet

Major type	Description						Binary	Shorthand

0		an unsigned integer					000_xxxxx	unsigned(#)

1		a negative integer					001_xxxxx	negative(#-1)

2		a byte string						010_xxxxx	bytes(n)

3		a text string (UTF-8)					011_xxxxx	text(n)

4		an array of data items					100_xxxxx	array(n)

5		a map of pairs of data items				101_xxxxx	map(n)

7		Floating-point numbers and values with no content	111_xxxxx	simple(v)

1 For negative(#-1), # represents the negative value minus 1. For example, negative(4) represents a value of -5.

5 For map(n), n represents number of key/value pairs.

Unsigned integer

Number		Hex

0 .. 15			00 .. 0F

16 .. 23			10 .. 17

24 .. 255		18 18 .. 18 FF

256 .. 65535		19 0100 .. 19 FFFF

Negative integer

Number		Hex

-1 .. -16			20 .. 2F

-17 .. -24		30 .. 37

-25 .. -256		38 18 .. 38 FF

-257 .. -65536		39 0100 .. 39 FFFF

Array

Length			Hex

0 .. 15			80 .. 8F

16 .. 23			90 .. 97

24 .. 255		98 18 .. 98 FF

256 .. 65535		99 0100 .. 99 FFFF

Indefinite		9F

Length represents number of key/value pairs.

Map

Length			Hex

0 .. 15			A0 .. AF

16 .. 23			B0 .. B7

24 .. 255		B8 18 .. B8 FF

256 .. 65535		B9 0100 .. B9 FFFF

Indefinite		BF

Length represents number of key/value pairs.

Floating point numbers and values with no content

5-Bit Value Semantics

20	false

21	true

22	null

23	undefined

25	IEEE 754 Half-Precision Float (16 bits follow)

26	IEEE 754 Single-Precision Float (32 bits follow)

27	IEEE 754 Double-Precision Float (64 bits follow)

31	"break" stop code for indefinite-length items

Examples:

| As JSON: { "t":2, "h":null } | As CBOR: A2 # map(2) 61 # text(1) 74 # "t" 02 # unsigned(2) 61 # text(1) 68 # "h" F6 # primitive(22) |
| --- | --- |